A Comparison Between Conventional Wheel Rail and Maglev Train Systems

Kerem Bal - 040210078

Kerem Karadeniz - 040210049

Asef Burak Doğan - 040210108

Zeynep Sena Güller - 040210221

Electronics & Communication Engineering, Faculty of Electrical and Electronics, Istanbul Technical University

ING 201A – Essentials of Research Paper Writing

Duygu Özkan

May 18, 2023

A Comparison Between Conventional Wheel Rail and Maglev Train Systems

Thesis Statement: Maglev and conventional wheel-rail trains can be compared in terms of four fundamental aspects such as technical features, economic feasibility, environmental impacts and energy efficiency.

- TECHNICAL FEATURES

- Effects of technical features to the passengers and surroundings

- Noise and vibration

- Intermodal compatibility

- Movement capability

- Speed and acceleration

- Resistance to motion

- Uphill gradients and curves

- ENERGY EFFICIENCY

- Power Consumption

- Less energy consumption

- More energy consumption

- Efficiency Factors

- Travel time

- Number of passengers

- ECONOMICAL FEASIBILITY

- Costs

- Construction Cost

- Operating Cost

- Long-term economic sustainability

- Reduction of petroleum consumption

- Economical advantage

- ENVIRONMENTAL IMPACTS

- Effects on climate change and ecology

- Less land usage and soil occupation

- Reduction of air pollution

- Effects on human health and comfort

- Decreasing the noise pollution level

- Harmless electromagnetic field exposure

Since the invention of steam locomotives, the global logistic network has adopted trains widely for both the movement of goods and the movement of people. Thanks to the developing technology, new train technologies such as Maglev, short for magnetic levitation, trains have emerged in the 20th century and the first Maglev systems were built in the early 21st century. The definition of Maglev is incredibly simple. This newly emerged technology is described as "A Maglev train car is just a box with magnets on the four corners" (Powell, n.d, as cited in U.S. Department of Energy, 2016).  Maglev trains use magnetic fields to simultaneously provide propulsion and levitation. It can be operated at around 500 km/h without danger due to its innovative technology and infrastructure. Maglev trains can provide transportation in a way that wheel-rail trains cannot compete in travel time. In this article, the two train types, Maglev and wheel-rail, are compared to determine which trains might be preferable in the future. As a consequence, Maglev and conventional wheel-rail trains can be compared in terms of four fundamental aspects such as technical features, economic feasibility, environmental impacts and energy efficiency.

The infrastructure of trains that are built with magnetic levitation technology is completely different from other railway systems, which creates some advantages and disadvantages. In general, the only competitor of Maglev trains is high-speed rail trains for decades due to the ease of comparison. One of the reasons why there is such distinctness is the technical differences between Maglev and other railway trains. It is confusing to consider what makes a train different from another. However, differences due to the technical specifications affect some of the keystones of a train such as the whole movement activity of the train, the experience of passengers and the influence of surroundings. To begin with, passengers can observe the difference between Maglev and high-speed rail systems in terms of noise and vibration due to the difference between railways and Maglev’s guided tracks. For example, Vuchic and Casello claimed that since the Maglev is not touching the guided track, less noise and vibration occur compared to high-speed rail trains while the train is moving according to indications (2002, p. 46). It is clear that Maglev trains have a great advantage in terms of noise and vibration as indicated in test results. The other effect of the technical features on the passengers and surroundings is intermodal compatibility. In fact, the primary addressees of this topic are governments. Each country prepares a cost and purpose calculation while considering Maglev projects; therefore, compatibility for networks is ensured. For example, Vuchic and Casello stated, although the standards of high-speed rail lines are built for different operations, allow their trains to travel on other existing railways. Therefore, the budget for construction, maintenance and other facilities can be significantly lower. For example, intercity express trains in Germany can operate between Frankfurt, Hannover and Hamburg at speeds up to 200 km/h while running on the new high-speed line (2002, p. 43). It can be understood that the purpose of Maglev trains must be set for either long or short-distance travel. In this way, while using the advantages of Maglev, we avoid its disadvantages. As a consequence, Maglev projects should be planned to operate in areas where there is no such railway to avoid disturbing the environment and optimize the budget.

Furthermore, standard wheel rails and Maglev trains are fundamentally different, thus their technical elements also differ. In fact, the roots of Maglev trains are grounded in shortening the travel time of trains, which is done by improving movement capability while using superconductivity. Firstly, one of the major superiors of the Maglev trains in comparison to high-speed rail trains is speed and acceleration with respect to movement capability. As an example, Lever implies that although the maximum acceleration of TGV (France's intercity high-speed rail service) is 0.39 m/s2 while its velocity is increasing from 0 to 16 m/s and it falls to 0.29 m/s2 at 50 m/s velocity, the maximum acceleration of Maglev is around 1.18 m/s2 at such low speeds. Additionally, the U.S. Maglev train can conserve its acceleration by 0.39 m/s2 at the velocity of 134 m/s (1998, p. 180). Another example is that, as Yaghoubi et al. stated, the acceleration of a Maglev train is virtually stable at 0.9 m/s2. While it takes around 5 minutes for the ICE (German Intercity Express) train to reach 300 km/h, its maximum speed, the Maglev train can claim its maximum speed, 450 km/h, in 3 minutes (2011, p. 6). As a result, it is clear that high-speed rail trains are not eligible to compete with Maglev trains with respect to maximum speed and acceleration. Secondly, motion resistance is the other comparison point under the umbrella of movement capability between Maglev and high-speed rail trains. Cassat and Bourquin indicated that both the lack of a pantograph and simpler bogies in the Maglev systems allow for superior streamlining due to the fact that those components provide around one-fourth of the aerodynamic resistance together (2011, p. 38-39). In conclusion, some of the technical specifications such as pantograph and bogies affect the movement capability of a train. It can be claimed that Maglev trains have a great advantage in motion resistance compared to conventional wheel-rail. Thirdly, the other superior of Maglev trains in comparison to high-speed rail trains is their capability on uphill gradients and curves. For instance, while high-speed rail trains run on uphill gradients and slopes with a maximum inclination of 3.5 - 4 percent, Maglev vehicles can conveniently climb 10 percent slopes. Broadly, Maglev trains can overcome inclines from 2.5 to 8 times more vertical than high-speed rail trains without slowing down. Additionally, compared to the same speed with high-speed rail, Maglev trains can corner 1.5 times tighter curves, both horizontally and vertically, and go significantly faster than high-speed rail trains over a curve with the same radius. As an example, a Maglev vehicle climbs up to 16°. The minimum curve radius of the Maglev’s guided track at 300 km/h is between 1,590 and 2,360 m, while the radius of railways is 3350 m, which is 1.4 – 2.1 times greater (AMG, 2002, Liu & Deng 2004, Dai, 2005, Jehle et al., 2006, Stephan & Fritz, 2006, Baohua et al., 2008, as cited in Yaghoubi et al., 2011). As can be understood from above, it is safe to say that Maglev’s superiority stands out in comparison with high-speed rail trains with respect to movement capability on uphill gradients and curves. To summarize the movement capability comparison of Maglev trains and high-speed rail trains; there is no doubt in determining the winner due to the fact that Maglev technology stands for increasing the net force acting on trains, which directly affects maximum speed, acceleration, motion resistance, uphill climbing and cornering.

Another significant decision point on whether to implement Maglev or wheel rail train as a rapid transportation system is energy efficiency. Depending on how a system is designed and operated, power usage of the system changes rapidly. Since Maglev trains travel above the rail track, it requires both levitation force and propulsion force to operate. This design brings positive and negative traits together. On one side, it needs a dedicated energy for levitation that wheel rail-based trains do not require. However, because Maglev trains are operated above the track less energy is needed to advance by eliminating rail friction. Thus, between the Maglev and wheel-rail trains one does not directly arise as a better solution for power consumption. To begin with, Maglev trains might be preferable based on power consumption by eliminating rail friction altogether. Friction caused by the rail contact is a crucial contributor in energy loss for trains that use conventional wheel rail trains. By removing friction out of the system, the trains operate against only the drag caused by the air. This specific trait makes it possible to build energy efficient Maglev systems. A comparison on power consumption of wheel-rail and Maglev systems was made by Prasad et al. (2019) in their study. It is stated that the specific energy consumption of a Maglev system is in between 45 and 54 Wh/pl/km and the specific energy consumption of wheel-rail system is in between 48.5 and 59 Wh/pl/km. The study shows that Maglev trains are almost 10 percent more efficient in terms of power consumption (see Table 1). Another example of energy efficient Maglev system is the Transrapid train system that operates in Germany between Hamburg and Berlin. Fritz et al. (2018) found that the Maglev system that travels at 330km/h maximum speed has 45 Wh/Pl/km energy consumption. On the contrary, the high-speed rail system (ICE) that travels on the same line has 31 % higher power consumption than the Maglev train. Also, when the Maglev system reaches the 430 km/h travel speed the energy consumption becomes only 7% more than the ICE despite having 30% higher travel speed for the same line. It can be seen that a Maglev based system can be more beneficial in terms of energy efficiency. On the contrary side, despite having eliminated the energy loss caused by the friction, Maglev trains might also be energy inefficient depending on the several factors. Fritz et al. (2018) estimated the specific energy consumption of Chuo Maglev line in Japan as 27 kWh/km and a high-speed train line as 22 kWh/km when operating around 330km/h speed based on the novel approach that takes effective area into the consideration when calculating the energy consumption. They also indicate that when operating between 430 to 500 km/h speed the specific energy consumption becomes 36 kWh/km for TR Maglev train and 85 kWh/km for Chuo Maglev train lines. It should be noted that even though there is no friction, a Maglev system can still be inefficient when it comes to the power usage. It can also be deduced that total power consumption of a train system does not solely depend on the energy loss caused by friction. As a result, a decision to implement Maglev or wheel rail train system should be made on solely by energy consumption. Depending on the specifics of the planned route, Maglev system can be beneficial from all aspects or not. It is safe to say that the new technology is a serious competitor to the wheel-rail in terms of energy consumption.

Secondly, the factors that influences the decision point based on power consumption can be broken down to categories such as travel time and passenger load. Maglev lines that are in sub optimal conditions in terms of travel time and passenger load could be prone to low energy efficiency while operating. One major factor that should thoroughly examined on a Maglev line is the travel time of the train. In Maglev trains, Suspension and propulsion are considered to be responsible for the most of the energy that are used for the journey. The relationship between the travel time and propulsion, suspension energy is explained by the Lai et al. (2020) as not unilaterally but in divisions of three parts. In general, it can be seen that total energy consumption decreases while time advances. The suspension energy steadily increases as the time passes and for the times between 350 and 600 seconds the propulsion consumption decreases with a great curve and flattens after the 600 seconds. For the later part of the journey suspension consumption becomes higher than the propulsion consumption (see Figure 1).  It can be shown in the that travel time is really important factor while considering the energy efficiency and should be examined thoroughly. Another important factor to look for is passenger load per departure of the train. Depending on the implementation carrying more passengers can yield to the less average power consumption. A study made by Janić (2021) that depicts the connections in the nonstop journey distances and the average energy consumption for certain highspeed systems shows that no matter the transportation system higher passengers yields to lower average energy consumption. Janic’s data also shows that this approach is consistent for Maglev, High Speed Rail, Hyperloop and air travel as well. In terms of power consumption Maglev trains can be suitable for a demand, given that the passenger load is elaborately considered (see Figure 3). To sum up, in order to design a well-functioning energy efficient Maglev train system, the decisions should be made within an optimal travel timeframe and demand for high passenger count.

The economic feasibility of Maglev trains and conventional trains can be compared in many respects and it can be decided which one is more feasible. Recently, thanks to the developing technology, it can be understood that Maglev trains are more economically feasible and logical than conventional trains, considering the costs and long-term economic sustainability. Also, nowadays, significant researches show that Maglev trains have a greater potential in terms of contribution to country's economy. Constructing complex systems such as Maglev trains and conventional trains incurs high costs. While trains are being constructed, many factors such as terrain, line length, materials used, number of stations affect the cost of construction. In particular, the operating and maintenance of these trains after they are built also cause costs. When calculations are made on these costs, it is possible to compare the costs of trains and decide which train type will be more viable and preferable for which region. To begin with construction cost of Maglev trains, the infrastructure, the levitation system and advanced technology of Maglev trains cause higher costs. Rose et al. (2015) stated that the news of the Maglev train with superconductor and linear motor technology, which is designed to be construct between Tokyo and central Japan, with a completion date of 2025, were declared by Japan Railways in the last month of 2007. According to this project, the cost of Maglev train is estimated to be slightly more than 5 trillion yen, approximately 45 billion US dollars. From this information it is clear that the average construction cost of a Maglev train (varies with different factors) is considerably high and it is economically feasible for rich countries. In addition, the process of operating these trains after they are constructed also causes costs. Kluehspies (2018) points out that conventional trains, using rails and wheels, need extensive and compact maintenance when operating, because with rising speed, wear occurs on the trains. However, although Maglev trains operate at huge speeds, they can operate nearly without any possibility of disruption or breakdown, that makes Maglev trains are more cost-effective. It is clearly understood as in this information that Maglev trains have higher potential in terms of long-term use and therefore more economically feasible. In brief, when these two train types are compared, it cannot be said that this is more economically feasible, but although the costs of Maglev trains are high when they are constructed, the low probability of deterioration in the process and the fact that they do not require much maintenance in the process, make Maglev a step forward in economic feasibility.

Apart from comparing the costs, one can compare the long-term economic sustainability of Maglev trains or conventional trains and accordingly decide which one is more economically feasible. Various factors are considered such as, consumption of petroleum and economic advantages, when calculating the long-term economic sustainability of trains. For example, nowadays, most conventional trains use fuel that obtained from petroleum, against this Maglev trains operate on an electromagnetic system. In addition, while petroleum is running out day by day in the world, fuel prices are increasing. Curtis (2009) outlines fuel depletion will result in an increase in fuel prices. As a result, stuffs movement costs will be rising up. This will happen before the effects of global warming are seen more clearly. In the light of this information, considering the future, it can be said that Maglev trains are more economically sustainable in the long-term due to the fact that they do not use fuel and fuel prices increase with the depleted fuel which is food of conventional trains. Another example, the Maglev trains can have a large economic impact in the regions where it is constructed. Internal and external migrations may occur towards the places where Maglev trains are constructed, both because it will facilitate transportation and because it is comfortable. Thus, economic activity can occur in that region. As indicated by Mao et al. (2008), Maglev trains can significantly boost the worth of the land along the line where they are built and assist ensure the economic development of that area. It can be understood that, the long-term economic advantages that Maglev trains can be created make Maglev trains more sensible to most areas than conventional trains. To sum up, it is clear that Maglev trains will be more economically sustainable in the long run because of the decreasing fuel day by day and in the regions where conventional trains are built, they will remain weaker than Maglev trains in terms of economic contribution. For these reasons, economically feasibility of Maglev trains is more logical considering the contributions it will make.

Another reason why Maglev trains, which are one of the newest and most sustainable projects of recent times (in terms of environmentally), are preferred especially in developed countries is that their environmental impacts are quite different from other trains. These differences can be roughly listed as effects on ecological balance and human health. First of all, it can be determined that the systems of Maglev trains make a positive contribution to the ecological balance because of the fact that they occupy less land and release air polluting gases at a lower level compared to conventional trains. To begin with, due to their technical system, Maglev trains use less space in the areas where they will be located. In this way, they interfere less with natural life and help protect the areas where living things live. For example, in a research covering all stops and facilities, fuel stations and technical systems, it is calculated that Maglev trains use less space. According to Sands (1993), the area of occupied by Transrapid (one of the Maglev trains located in Germany) is 246 m2, while that of the other trains is 430 m2 (see Table 2). In other words, it can be clearly understood that Transrapid occupies slightly more than 50 percent of the other trains it competes with. Therefore; it can be said that the countries' preference for Maglev trains instead of conventional trains provides the ecological balance and reduction of occupation of natural areas. In addition, the fact that the fuel consumed is different compared to the competing conventional trains provides Maglev trains emit less polluting gases. Therefore; it causes less air pollution. As an example, according to Sands' study, although Transrapid emits 15000 mg/seat/km of CO2 at 300 km/h, Intercity Express (the high-speed wheel train that travels through all of Germany's metropolitan cities, ICE in short) emits 25000 mg/seat/km of CO2 at the same speed (see Table 3). Based on this data, it can be determined that 60% of ICE's gas emissions are released by Transrapid (1993). As a result, it can be determined that Maglev releases less CO2 and similar polluting gases and also, significantly reduces the levels of factors that cause air pollution compared to high-speed trains. Consequently, all these examples prove that when Maglev trains is preferred by countries instead of conventional or high-speed trains, it occupies less natural area and contributes to the protection of ecological balance, while at the same time reducing the emission level of CO2 and similar polluting gases that are released into the atmosphere and cause climate change.

Secondly, due to the fact that Maglev trains emit much less noise than their competitors and the electromagnetic field to which passengers are exposed is at a very low level, they cause less harm to the environment and human health. At the beginning, the friction force acting on the Maglev trains is reduced as a result of levitation while in moving. By performing this, the comfort level is increased while the level of noise that may irritate the other passengers and the surroundings is reduced. For example, based on researches by Chen et al. (2007), it is proved by their survey that the Shanghai Maglev train is 4-8 dB quieter than high-speed wheel trains. Furthermore, according to Wang et al. (2010), other researchers who study on transportation industry, the noise level which is emitted by Maglev and high-speed wheel trains at 250 km/h was evaluated 73 and 83 units, respectively (see Table 4). In the light of these information, the fact that the Maglev has a low friction force because there is no wheel-rail system in its system explains that it is quieter than other trains. All these researches show that since the Maglev is at a more tolerable noise level, it can be inferred that it provides a more comfortable travel. Moreover, Maglev trains (which produces an electromagnetic field while moving) does not create a condition that will adversely affect both the passengers and the people nearby. Because it does not generate magnetic fields that are hazardous to human health. To give an example to explain how harmless the Maglev trains’ magnetic field is, as indicated by Kircher's research, with respect to the frequency, the maximum induction value can reach more than 25 µT and is barely below the floor, while the magnetic field strength at central Europe is 48 µT (2018). Furthermore, in another study, Sands determined that the magnetic field was measured at 0.1 Gauss on the Maglev ground, 0.02 Gauss at the seat level and 0.02 Gauss on the side of the vehicle (see Table 5). However, since the hair dryer emits 9.7 Gauss and the video monitor emits 0.3 Gauss. (1993). It is confirmed that Maglev trains, a technology that used extensively magnetism, will not have a significant negative impact on passengers, particularly locals. However, it is unknown how long-term biological impacts will be caused. In conclusion, compared to other competitor trains such as conventional and high-speed wheel trains, Maglev actually minimizes the elements that cause noise pollution as much as possible and although it creates a magnetic field, its effect on humans is almost negligible.

In conclusion, developing technology leads to novel and competitive transportation options, such as Maglev trains. Maglev trains and wheel-rail conventional trains can be assessed revolving around technical features, economic feasibility, environmental impacts and energy efficiency. It is safe to say that Maglev shows promising outcomes even though it has been tested in large scope for only two decades while its main competitor high-speed wheel rail trains have been building for two centuries. Although these two types of trains are not superior to each other and both Maglev and wheel-rail have their special assets, when compared to today's conditions, Maglev trains show great potential in terms of transportation in the future with the development and investments in addition to Maglev’s innovative technology.

References

Cassat, A., & Bourquin, V. (2011, December). Maglev–worldwide status and technical review. In Proceedings of the Conference “Électrotechnique du Futur (pp. 14-15). https://www.researchgate.net/publication/236993225_MAGLEV_-_Worldwide_Status_and_Technical_Review

Chen, X., Tang F., Huang, Z. & Wang, G. (2007) High-speed maglev noise impacts on residents: A case study in Shanghai. Transportation Research Part D: Transport and Environment, 12(6), 437-448. https://doi.org/10.1016/j.trd.2007.05.006

Curtis, F. (2009). Peak Globalization: Climate Change, Oil Depletion and Global Trade, Ecological Economics, 69(2), 427-434.  https://doi.org/10.1016/j.ecolecon.2009.08.020

Fritz, E., Blow, L., Kluhspies, J., Kircher, R., & Witt, M. H. (2018). Energy consumption of track-based high-speed trains: maglev systems in comparison with wheel-rail systems. Transportation Systems And Technology, 4(3 suppl. 1), 134-155. https://doi.org/10.17816/transsyst201843s1134-155

Huang, R., Mao, B., & Jia, S. (2008). Potential Applications of Maglev Railway Technology in China, Transportation Systems Engineering and Information Technology, 8(1), 29-39.  https://doi.org/10.1016/S1570-6672(08)60007-0

Janić, M. (2021). Estimation of direct energy consumption and CO2 emission by high speed rail, transrapid maglev and hyperloop passenger transport systems. International Journal of Sustainable Transportation, 15(9), 696-717.  https://doi.org/10.1080/15568318.2020.1789780

Kircher R, Klühspies J, Palka R, et al. (2018) Electromagnetic fields related to high speed transportation systems. Transportation Systems and Technology, 4(2), 152-166. https://doi.org/10.17816/transsyst201842152-166

Kluehspies, J. (2018). Maglev Trends in Public Transport: The Perspectives of Maglev Transportation Systems. Transportation Systems and Technology, 4(3s1),  5-12. https://doi.org/10.17816/transsyst201843s15-12

Lai, Q., Liu, J., Haghani, A., Meng, L., & Wang, Y. (2020). Energy-efficient speed profile optimization for medium-speed maglev trains. Transportation Research Part E: Logistics and Transportation Review, 141, 102007. https://doi.org/10.1016/j.tre.2020.102007

Leung, E. M., Peterson, D. E., Rose, C. R. (2008). Implementation of Cargo MagLev in the United States. U.S. Department of Energy Office of Scientific and Technical Information, No. 081, n.p.   https://www.osti.gov/biblio/964981

Lever, J. H. (1998). Technical assessment of maglev system concepts (No. CRREL-SR-98-12). United States. Army Corps of Engineers.

Prasad, N., Jain, S., & Gupta, S. (2019). Electrical components of maglev systems: Emerging trends. Urban Rail Transit, 5, 67-79. https://doi.org/10.1007/s40864-019-0104-1

Sands, B. (1993). The german magnetic levitation train (Transrapid). Built Environment (1978-), 19(3/4), 244-256. https://www.jstor.org/stable/23288580

Vuchic, V. R., & Casello, J. M. (2002). An evaluation of maglev technology and its comparison with high speed rail. Transportation Quarterly, 56(2), 33. https://repository.upenn.edu/ese_papers/820/

Wang, C. & Wang, K. (2010). A study on environmental impact of high speed maglev traffic engineering [Conference session]. International Conference on Management and Service Science, Wuhan, China. https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=5576917

Yaghoubi, H., Barazi, N., Kahkeshan, K., Zare, A., & Ghazanfari, H. (2011, October). Technical comparison of maglev and rail rapid transit systems. In The 21st International Conference on Magnetically Levitated Systems and Linear Drives (MAGLEV 2011). http://www.maglev.ir/eng/documents/papers/conferences/maglev2011/DPO-05.pdf

Table 1

Different features of Maglev and wheel rail systems.

Note. From “Electrical Components of Maglev Systems: Emerging Trends,” by N. Prasad, S. Jain, & S. Gupta, 2019, Urban Rail Transit, p. 70, https://doi.org/10.1007/s40864-019-0104-1

Table 2

Comparison Of Transrapid Land Use To That Of Conventional Rail And Freeways In Germany

Note. From “The german magnetic levitation train (Transrapid)” by B. Sands, 1993, p.8. https://www.jstor.org/stable/23288580

Table 3

Comparison Of Emissions In Mg/Seat-Km For Various Transportation Systems

Note. From “The german magnetic levitation train (Transrapid)” by B. Sands, 1993, p.10. https://www.jstor.org/stable/23288580

Table 4:

Note. From “The german magnetic levitation train (Transrapid)” by C. Wang & K. Wang, 2010, p.3. https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=5576917

Table 5

Magnetic Field Measurements From The Transrapid

Note. From “The german magnetic levitation train (Transrapid)” by B. Sands, 1993, p.9. https://www.jstor.org/stable/23288580

Figure 1

The relationship between the travel time and energy consumption.

Note. From “Energy-efficient speed profile optimization for medium-speed

maglev trains,” by Q. Lai, J. Liu, A. Haghani, L. Meng & Y. Wang, 2020, Transportation Research Part E: Logistics and Transportation Review, p. 20, https://doi.org/10.1016/j.tre.2020.102007

Figure 2

The connection of average energy consumption and the distance of nonstop journey for high speed systems. HSR: High speed rail, TRM: Transrapid Maglev, HL: Hyperloop

Note. From “Estimation of direct energy consumption and CO2

emission by high speed rail, transrapid maglev and

hyperloop passenger transport systems,” by M. Janić, 2021, p. 710, https://doi.org/10.1080/15568318.2020.1789780